CMPG 325 – Computer Networks

Client Implementation Review

Milestone 2 – Kamogelo Insurance Brokers

| Student | Motheo Motsumi – student number 41961366 |
| --- | --- |
| Project ID / Client ID | CMPG325-2026-089 / CLI-089 |
| Client | Kamogelo Insurance Brokers (Vryburg) – Professional Services |
| Assigned addressing block | 192.168.40.0/24 |
| Assigned networking challenge | Wireless LAN (AP integration & coverage) – Foundational |
| Change request | CR15 – second Internet connection added for resilience |
| Milestone | Milestone 2 – Client Implementation Review (due 2 October 2026, 23:55) |
| GitHub portfolio | https://github.com/MotheoMotsumi/CMPG325-Project-CLI-089 |

# 1. Submission overview

This document accompanies the Milestone 2 submission. It describes what was implemented in Packet Tracer, how it follows the Milestone 1 design, and the evidence that it works. The four items required for review are listed below.

| # | Required item | Status | Where to find it |
| --- | --- | --- | --- |
| 1 | Working Packet Tracer file | Submitted | 41961366_CMPG325_M2.pkt (uploaded to eFundi and stored in the repository under packet-tracer/) |
| 2 | Assigned feature implemented | Complete | Wireless LAN – AP integration & coverage: Section 4 |
| 3 | Testing evidence | Complete | Section 7 of this document; full screenshots in Milestone2_Testing_Evidence_41961366.docx and in the repository under evidence/screenshots/ |
| 4 | Updated GitHub portfolio | Updated | https://github.com/MotheoMotsumi/CMPG325-Project-CLI-089 |

# 2. Implementation summary

The network was built exactly as a three-tier design: two edge routers (one per ISP), a core router, a core switch and four access switches. The table maps each Milestone 1 design element to what was built.

| Milestone 1 design element | Milestone 2 implementation |
| --- | --- |
| Departmental segmentation | VLANs 10 (Sales), 20 (Admin), 30 (Boardroom) and 99 (Server), subnetted from 192.168.40.0/24 with VLSM. |
| Inter-VLAN routing on R3-CORE | Router-on-a-stick: subinterfaces Gi0/0.10, .20, .30 and .99 on an 802.1Q trunk to SW1-CORE. Milestone 1 left the gateway placement open (router-on-a-stick or SVIs on SW1-CORE; its address table labels the gateways "SVI"). Router-on-a-stick on R3-CORE was implemented, using the same gateway addresses. |
| SW1-CORE as the core switch | Cisco 3650-24PS used for trunking to the four access switches; routing is done on R3-CORE. |
| Boardroom: dedicated wired and wireless presentation ports | SW4-BOARD Fa0/1 is the wired presentation port (Board-Presenter-PC) and Fa0/2 connects AP1-BOARD. Both are in VLAN 30. |
| Assigned challenge: Wireless LAN (AP integration & coverage) | AP1-BOARD with SSID KAMOGELO-BOARD (WPA2-PSK, AES), wireless laptop, phone and tablet receiving DHCP from the boardroom pool (Section 4). |
| CR15: second Internet connection | R2-EDGE-BAK and ISP-B added beside R1-EDGE-PRI and ISP-A, with floating static default routes on R3-CORE (Section 5). |
| Address plan | DHCP pools on R3-CORE for Sales, Admin and Boardroom; static addresses for the servers and the presenter PC; /30 links for the WAN side. |

Two items go beyond the Milestone 1 text and are documented here for completeness: the addressing of the two ISP links (outside the client block) and the boardroom isolation ACL (Section 6).

## 2.1 Differences from the Milestone 1 diagrams

The implementation keeps every subnet, VLAN, role and link of the Milestone 1 design. The differences below are implementation details that the diagrams showed differently.

| Item | Milestone 1 diagrams | Implemented in Packet Tracer |
| --- | --- | --- |
| Router model and ports | Cisco 4331 ISR (Gi0/0/x) | Cisco 2911 (Gi0/x): provides the three gigabit ports R3-CORE needs; roles unchanged |
| Core switch | Catalyst 3560 multilayer switch | 3650-24PS, used for 802.1Q trunking to the access switches |
| WAN link host addresses | Logical diagram: R1 .73 and core .74; R2 .77 and core .78 | R3-CORE .73 and .77; R1 .74 and R2 .78. The subnets (192.168.40.72/30 and .76/30) are unchanged; only the host assignment inside each /30 differs |
| Boardroom SSID | KIB-BOARD (label on the logical diagram) | KAMOGELO-BOARD |
| Gateway placement | Router-on-a-stick or SVIs left open | Router-on-a-stick on R3-CORE |
| End devices | Three Sales PCs, two Admin PCs plus a manager laptop, one file/print server, one tablet and one laptop on Wi-Fi | Two PCs per department, a File-Server and a Print-Server, and a wireless laptop, phone and tablet (a representative sample) |

# 3. Topology and addressing

Figure 1 – Implemented topology (full-size screenshot in the evidence document, figure T16)

## 3.1 Devices

| Device | Model | Role |
| --- | --- | --- |
| R1-EDGE-PRI | 2911 | Primary edge router (ISP-A), NAT overload |
| R2-EDGE-BAK | 2911 | Backup edge router (ISP-B), NAT overload |
| R3-CORE | 2911 | Inter-VLAN routing, DHCP server, default routes, boardroom ACL |
| SW1-CORE | 3650-24PS | Core switch; trunks to the access switches |
| SW2-SALES, SW3-ADMIN, SW4-BOARD, SW5-SRV | 2960-24TT | Access switches (Sales, Admin, Boardroom, Server) |
| AP1-BOARD | Access Point-PT | Boardroom wireless access point |
| ISP-A, ISP-B | 2911 | Simulated ISPs; each has loopback 8.8.8.8 as a stand-in Internet host |
| Sales-PC1/2, Admin-PC1/2, Board-Presenter-PC, File-Server, Print-Server | PC / Server | Wired end devices |
| Board-Laptop, Board-Phone, Board-Tablet | Laptop / Smartphone / Tablet | Wireless boardroom clients |

## 3.2 Cabling

| From | To |
| --- | --- |
| R3-CORE Gi0/0 | SW1-CORE Gi1/0/1 (trunk) |
| SW1-CORE Gi1/0/2, 3, 4, 5 | SW2-SALES, SW3-ADMIN, SW4-BOARD, SW5-SRV – each on Gi0/1 (trunks) |
| R3-CORE Gi0/1 | R1-EDGE-PRI Gi0/0 (192.168.40.72/30) |
| R3-CORE Gi0/2 | R2-EDGE-BAK Gi0/0 (192.168.40.76/30) |
| R1-EDGE-PRI Gi0/1 | ISP-A Gi0/0 (203.0.113.0/30) |
| R2-EDGE-BAK Gi0/1 | ISP-B Gi0/0 (198.51.100.0/30) |
| SW2-SALES Fa0/1–2 | Sales-PC1, Sales-PC2 |
| SW3-ADMIN Fa0/1–2 | Admin-PC1, Admin-PC2 |
| SW4-BOARD Fa0/1 / Fa0/2 | Board-Presenter-PC / AP1-BOARD (Port 0) |
| SW5-SRV Fa0/1–2 | File-Server, Print-Server |

## 3.3 Addressing

| Segment | Network | Mask | Gateway | Assignment |
| --- | --- | --- | --- | --- |
| VLAN 10 Sales | 192.168.40.0/27 | 255.255.255.224 | .1 (Gi0/0.10) | DHCP |
| VLAN 20 Admin | 192.168.40.32/28 | 255.255.255.240 | .33 (Gi0/0.20) | DHCP |
| VLAN 30 Boardroom | 192.168.40.48/28 | 255.255.255.240 | .49 (Gi0/0.30) | DHCP .51–.62; presenter PC static .50 |
| VLAN 99 Server | 192.168.40.64/29 | 255.255.255.248 | .65 (Gi0/0.99) | Static: File-Server .66, Print-Server .67 |
| WAN R3–R1 | 192.168.40.72/30 | 255.255.255.252 | R3 .73, R1 .74 | Static |
| WAN R3–R2 | 192.168.40.76/30 | 255.255.255.252 | R3 .77, R2 .78 | Static |
| ISP-A link | 203.0.113.0/30 | 255.255.255.252 | ISP-A .1, R1 .2 | Static (outside client block) |
| ISP-B link | 198.51.100.0/30 | 255.255.255.252 | ISP-B .1, R2 .2 | Static (outside client block) |

## 3.4 DHCP pools (R3-CORE)

| Pool | Network | Excluded | Leases issued from |
| --- | --- | --- | --- |
| SALES | 192.168.40.0/27 | .1 (gateway) | .2 – .30 |
| ADMIN | 192.168.40.32/28 | .33 (gateway) | .34 – .46 |
| BOARD | 192.168.40.48/28 | .49 (gateway), .50 (presenter PC) | .51 – .62 |

All pools hand out the gateway of their own VLAN and DNS server 8.8.8.8.

# 4. Assigned feature: Wireless LAN (AP integration & coverage)

## 4.1 Design

One access point serves the boardroom. It is connected to the boardroom access switch on a port in VLAN 30, so wireless clients share the same VLAN, gateway and DHCP pool as the wired presentation port. This satisfies the design constraint that the boardroom has both a dedicated wireless and a dedicated wired presentation connection.

## 4.2 Access point configuration

| Setting | Value |
| --- | --- |
| Device | AP1-BOARD (Access Point-PT) |
| Wired uplink | Port 0 to SW4-BOARD Fa0/2 (access port, VLAN 30) |
| Wireless interface | Port 1, port status on |
| SSID | KAMOGELO-BOARD |
| Authentication / encryption | WPA2-PSK / AES |
| 2.4 GHz channel | 1 |
| Passphrase | Set on the AP and on each client; not reproduced in this document |

Figure 2 – AP1-BOARD wireless settings (passphrase masked)

## 4.3 Clients

| Client | Address (DHCP) | Gateway | Notes |
| --- | --- | --- | --- |
| Board-Tablet | 192.168.40.51/28 | 192.168.40.49 | Wireless0, WPA2-PSK |
| Board-Phone | 192.168.40.52/28 | 192.168.40.49 | Wireless0, WPA2-PSK |
| Board-Laptop | 192.168.40.53/28 | 192.168.40.49 | Wireless module fitted, WPA2-PSK |
| Board-Presenter-PC (wired) | 192.168.40.50/28 (static) | 192.168.40.49 | SW4-BOARD Fa0/1 |

## 4.4 Results

- Association and addressing (T03, T05, T16): all three wireless clients associate with AP1-BOARD and receive addresses from the BOARD pool with gateway 192.168.40.49.

- Wired/wireless integration (T06): the wireless laptop reaches the wired presenter PC with TTL 128, which shows the traffic stayed inside VLAN 30 and was not routed.

- Reachability beyond the VLAN (T07, T08): the laptop reaches the File-Server (routed by R3-CORE) and the Internet host 8.8.8.8 through the primary ISP.

- Security (T04b): a client configured with a wrong WPA2 passphrase fails to associate (IPv4 0.0.0.0) and reconnects with 192.168.40.51 as soon as the correct passphrase is entered.

- Coverage: the three clients are placed in the boardroom area of the workspace and each holds an active wireless link to the AP (T16). Packet Tracer’s range setting is a simulation parameter, not an RF survey.

# 5. Second Internet connection (CR15)

## 5.1 Design

A second edge router, R2-EDGE-BAK, connects to ISP-B and to R3-CORE through its own /30 link. Both edge routers translate the internal block with NAT overload on their ISP-facing interface. R3-CORE has two default routes: the primary via R1 with the default administrative distance, and a floating static route via R2 with administrative distance 10 that only enters the routing table when the primary route is withdrawn.

| Route on R3-CORE | Next hop | Purpose |
| --- | --- | --- |
| 0.0.0.0/0 (AD 1) | 192.168.40.74 (R1-EDGE-PRI) | Primary Internet path via ISP-A |
| 0.0.0.0/0 (AD 10) | 192.168.40.78 (R2-EDGE-BAK) | Backup Internet path via ISP-B (floating static) |

## 5.2 Failover test

| Step | Action | Observed result (evidence) |
| --- | --- | --- |
| 1 | Baseline: tracert 8.8.8.8 from Sales-PC1 | Path 192.168.40.1, 192.168.40.74, 8.8.8.8 (T11) |
| 2 | Bring the primary path down (R1-EDGE-PRI Gi0/0 shut down; R3-CORE Gi0/1 line protocol drops) | R3 default route becomes [10/0] via 192.168.40.78 (T12) |
| 3 | tracert 8.8.8.8 from Sales-PC1; ping from R2 | Path via 192.168.40.78; R2 reaches ISP-B and 8.8.8.8 (T13) |
| 4 | Restore the primary path (no shutdown) | Default route returns to [1/0] via 192.168.40.74 (T14) |
| 5 | NAT table on R1 after traffic from Sales-PC1 | Translations to 203.0.113.2 (T15) |

## 5.3 Scope and limitation

Floating static routes withdraw the primary route when the R3–R1 link or R1 itself fails. They do not detect an outage further upstream (for example, ISP-A failing while R1 stays up). Route tracking with IP SLA was not available in the Packet Tracer version used (the show track and show ip sla commands were rejected), so this limitation is accepted and recorded in Section 8.

# 6. Boardroom isolation (ACL)

Milestone 1 describes the boardroom as isolated from the rest of the network, but plain inter-VLAN routing would leave it reachable. An extended ACL on R3-CORE enforces the isolation while still letting boardroom devices use the server VLAN and the Internet.

ip access-list extended BOARD-IN

deny ip 192.168.40.48 0.0.0.15 192.168.40.0 0.0.0.31

deny ip 192.168.40.48 0.0.0.15 192.168.40.32 0.0.0.15

permit ip any any

interface gigabitEthernet 0/0.30

ip access-group BOARD-IN in

- Boardroom to Sales or Admin: blocked; the router answers "Destination host unreachable" (T09).

- Sales to boardroom: times out, because the boardroom’s replies are dropped by the same ACL (T09).

- Boardroom to its own VLAN, the File-Server and 8.8.8.8: still works (T09).

- Sales to Admin: unaffected (T09).

- The ACL hit counters (8, 4 and 8 matches) match the traffic generated during the test (T09).

# 7. Testing summary

Every test below passed. A single lost packet on the first ping to a new destination is normal: it is spent on ARP resolution. The matching screenshots, with explanations, are in Milestone2_Testing_Evidence_41961366.docx and in the repository under evidence/screenshots/.

| ID | Test | Expected result | Result |
| --- | --- | --- | --- |
| T01 | VLANs created and access ports assigned on all five switches | VLANs 10/20/30/99 exist; access ports sit in the correct VLAN | Pass |
| T02 | Trunks between SW1-CORE and the access switches | Gi0/1 trunking on each access switch; Gi1/0/2–5 trunking on SW1-CORE | Pass |
| T03 | DHCP leases issued by R3-CORE to wired and wireless clients | Leases inside the Sales, Admin and Boardroom pools | Pass |
| T04a | AP1-BOARD wireless security configuration | SSID KAMOGELO-BOARD, WPA2-PSK, AES, channel 1 | Pass |
| T04b | Client with a wrong passphrase is rejected, then reconnects with the correct one | No association (0.0.0.0), then 192.168.40.51 after correction | Pass |
| T05 | Wireless clients addressing | 192.168.40.51–53, mask 255.255.255.240, gateway 192.168.40.49 | Pass |
| T06 | Wireless laptop to wired presenter PC (192.168.40.50) | Reachable inside VLAN 30 (TTL 128, no routing) | Pass |
| T07 | Wireless laptop to File-Server (192.168.40.66) | Reachable, routed once by R3-CORE (TTL 127) | Pass |
| T08 | Wireless laptop to Internet host 8.8.8.8 | Reachable through the primary ISP path | Pass |
| T09 | Boardroom isolation ACL BOARD-IN | Boardroom blocked to/from Sales and Admin; server and Internet still reachable | Pass |
| T10 | Inter-VLAN routing from Sales-PC1 (before the ACL) | Reaches Admin, server and boardroom hosts (TTL 127) | Pass |
| T11 | Internet path under normal operation | tracert 8.8.8.8 via 192.168.40.1, 192.168.40.74 (R1) | Pass |
| T12 | Failover: default route when the primary path is down | S* 0.0.0.0/0 [10/0] via 192.168.40.78 | Pass |
| T13 | Traffic carried by the backup path | tracert via 192.168.40.78; R2 reaches ISP-B and 8.8.8.8 | Pass |
| T14 | Restore: default route returns to the primary | S* 0.0.0.0/0 [1/0] via 192.168.40.74 | Pass |
| T15 | NAT overload on the primary edge router | Translations to 203.0.113.2 on R1-EDGE-PRI | Pass |
| T16 | Wireless clients associated with AP1-BOARD in the boardroom | Laptop, phone and tablet each show an active wireless link | Pass |

Separate screenshots of the edge-router link checks and of the R2 NAT table are not included; backup-path NAT is shown to work by T13 (the Internet host is reachable through R2), and NAT overload is shown on R1 by T15.

# 8. Limitations

- Failover relies on floating static routes: it detects local link or router failure but not an ISP outage beyond R1 (Section 5.3).

- A single standalone access point covers the boardroom; there is no wireless LAN controller.

- Packet Tracer’s wireless range is a simulation setting and not a radio-frequency survey.

- The Internet is simulated by a loopback address (8.8.8.8) on each ISP router.

- No device hardening (enable secrets, SSH) was applied for this milestone.

# 9. GitHub portfolio

Repository: https://github.com/MotheoMotsumi/CMPG325-Project-CLI-089

| Folder / file | Contents |
| --- | --- |
| README.md | Project summary, addressing, feature description and test summary |
| docs/ | Milestone 1 design review, this implementation review, the testing evidence document |
| packet-tracer/ | 41961366_CMPG325_M2.pkt |
| configs/ | Final-state configuration, one file per router and switch |
| evidence/screenshots/ | All test screenshots, named by test ID (T01 … T16) |

# Appendix A – Configuration reference

The commands below are the configuration applied to each device in its final state. The same configuration is stored as one file per device in configs/ in the repository.

## R3-CORE

hostname R3-CORE

no ip domain-lookup

interface gigabitEthernet 0/0

description Trunk to SW1-CORE

no shutdown

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.40.1 255.255.255.224

interface gigabitEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.40.33 255.255.255.240

interface gigabitEthernet 0/0.30

encapsulation dot1Q 30

ip address 192.168.40.49 255.255.255.240

ip access-group BOARD-IN in

interface gigabitEthernet 0/0.99

encapsulation dot1Q 99

ip address 192.168.40.65 255.255.255.248

interface gigabitEthernet 0/1

ip address 192.168.40.73 255.255.255.252

no shutdown

interface gigabitEthernet 0/2

ip address 192.168.40.77 255.255.255.252

no shutdown

ip dhcp excluded-address 192.168.40.1

ip dhcp excluded-address 192.168.40.33

ip dhcp excluded-address 192.168.40.49 192.168.40.50

ip dhcp pool SALES

network 192.168.40.0 255.255.255.224

default-router 192.168.40.1

dns-server 8.8.8.8

ip dhcp pool ADMIN

network 192.168.40.32 255.255.255.240

default-router 192.168.40.33

dns-server 8.8.8.8

ip dhcp pool BOARD

network 192.168.40.48 255.255.255.240

default-router 192.168.40.49

dns-server 8.8.8.8

ip route 0.0.0.0 0.0.0.0 192.168.40.74

ip route 0.0.0.0 0.0.0.0 192.168.40.78 10

ip access-list extended BOARD-IN

deny ip 192.168.40.48 0.0.0.15 192.168.40.0 0.0.0.31

deny ip 192.168.40.48 0.0.0.15 192.168.40.32 0.0.0.15

permit ip any any

## R1-EDGE-PRI

hostname R1-EDGE-PRI

no ip domain-lookup

interface gigabitEthernet 0/0

ip address 192.168.40.74 255.255.255.252

ip nat inside

no shutdown

interface gigabitEthernet 0/1

ip address 203.0.113.2 255.255.255.252

ip nat outside

no shutdown

ip route 0.0.0.0 0.0.0.0 203.0.113.1

ip route 192.168.40.0 255.255.255.0 192.168.40.73

access-list 1 permit 192.168.40.0 0.0.0.255

ip nat inside source list 1 interface gigabitEthernet 0/1 overload

## R2-EDGE-BAK

hostname R2-EDGE-BAK

no ip domain-lookup

interface gigabitEthernet 0/0

ip address 192.168.40.78 255.255.255.252

ip nat inside

no shutdown

interface gigabitEthernet 0/1

ip address 198.51.100.2 255.255.255.252

ip nat outside

no shutdown

ip route 0.0.0.0 0.0.0.0 198.51.100.1

ip route 192.168.40.0 255.255.255.0 192.168.40.77

access-list 1 permit 192.168.40.0 0.0.0.255

ip nat inside source list 1 interface gigabitEthernet 0/1 overload

## ISP-A and ISP-B

! ISP-A

hostname ISP-A

interface gigabitEthernet 0/0

ip address 203.0.113.1 255.255.255.252

no shutdown

interface loopback 0

ip address 8.8.8.8 255.255.255.255

! ISP-B

hostname ISP-B

interface gigabitEthernet 0/0

ip address 198.51.100.1 255.255.255.252

no shutdown

interface loopback 0

ip address 8.8.8.8 255.255.255.255

## Switches

All five switches carry the same VLAN definitions:

vlan 10

name SALES

vlan 20

name ADMIN

vlan 30

name BOARDROOM

vlan 99

name SERVER

SW1-CORE (3650-24PS)

hostname SW1-CORE

interface range gigabitEthernet 1/0/1-5

switchport trunk encapsulation dot1q

switchport mode trunk

SW2-SALES (2960-24TT) – SW3-ADMIN is identical with VLAN 20 and SW5-SRV with VLAN 99

hostname SW2-SALES

interface range fastEthernet 0/1-2

switchport mode access

switchport access vlan 10

interface gigabitEthernet 0/1

switchport mode trunk

SW4-BOARD (2960-24TT)

hostname SW4-BOARD

interface fastEthernet 0/1

description Wired presentation port - Board-Presenter-PC

switchport mode access

switchport access vlan 30

interface fastEthernet 0/2

description Wireless presentation - AP1-BOARD

switchport mode access

switchport access vlan 30

interface gigabitEthernet 0/1

switchport mode trunk

## End-device settings

| Device | Address / mask | Gateway |
| --- | --- | --- |
| Sales-PC1, Sales-PC2, Admin-PC1, Admin-PC2 | DHCP | From the DHCP pool |
| Board-Presenter-PC | 192.168.40.50 / 255.255.255.240 (static) | 192.168.40.49 |
| File-Server | 192.168.40.66 / 255.255.255.248 (static) | 192.168.40.65 |
| Print-Server | 192.168.40.67 / 255.255.255.248 (static) | 192.168.40.65 |
| Board-Laptop, Board-Phone, Board-Tablet | DHCP (BOARD pool) | 192.168.40.49 |